FMI MCS10 J1939
Communication Protocol

Version 1.2 - Message Structure Clarified Edition

| Item | Description |
| --- | --- |
| Product | FMI MCS10 |
| Protocol | SAE J1939, extended CAN identifier |
| Default baud rate | 250 kbps |
| Default MCS10 ECU address | 0x7F |
| Default master address used in examples | 0x00 |

Revision History

| Version | Description |
| --- | --- |
| V1.1 | Reorganized document structure and clarified CAN message structure, command frame layout, request/response rules, and broadcast data parsing. |

Table of Contents

1. Communication Overview

2. Message Structure Overview

3. CAN Identifier Rules

4. Parameter Configuration Protocol (Privilege A)

5. Broadcast Data Protocol (Privilege B)

6. Quick Configuration Procedures

7. Appendix: Encoding Notes and Example Values

1. Communication Overview

1.1 Purpose and Scope

This document describes the FMI MCS10 J1939 communication interface. It explains how to build parameter configuration frames, how to identify request and response frames, and how to parse broadcast data frames.

1.2 Default Communication Settings

| Parameter | Default Value | Remark |
| --- | --- | --- |
| CAN baud rate | 250 kbps | Can be changed through BasicParam / CAN baudrate. |
| MCS10 ECU address | 0x7F | Used as destination address in configuration requests and source address in responses/broadcasts. |
| CAN frame format | 29-bit identifier | Extended CAN frame. |
| Payload length | 8 bytes | All examples in this document use DLC = 8. |
| Byte order | Little-endian | Multi-byte payload fields are sent low byte first. |

1.3 Two Types of J1939 Messages

| Message Type | Purpose | Direction | How to Identify |
| --- | --- | --- | --- |
| Parameter Configuration (Privilege A) | Read or write device parameters, reset/save/apply parameters. | Master <-> MCS10 | PGN = 0xEF00. DATA[0] is ctrl, DATA[1] is subcmd. |
| Broadcast Data (Privilege B) | MCS10 periodically outputs measurement data such as Euler angle, acceleration, gyro, quaternion, etc. | MCS10 -> bus | PGN = 0xFF00 to 0xFF06. DATA[0..7] are measurement fields, not ctrl/subcmd. |

Important: configuration frames and broadcast frames have different DATA meanings. In configuration frames, DATA[0] is the control byte. In broadcast frames, DATA[0] is the first data byte of the selected PGN.

2. Message Structure Overview

2.1 One CAN Frame = CAN ID + DATA

Every J1939 frame in this document should be read in two layers: the 29-bit CAN identifier tells the message direction/type, and the 8-byte DATA field carries either a command or measurement data.

| Layer | Field | Meaning | Customer Reading Method |
| --- | --- | --- | --- |
| Layer 1 | 29-bit CAN ID | Priority, PGN, destination/source address. | First confirm whether this is a configuration frame (0xEF00) or a broadcast data frame (0xFFxx). |
| Layer 2 | DATA[0..7] | Command bytes or sensor data bytes. | Interpret DATA according to the PGN. Do not use the configuration DATA format for broadcast messages. |

2.2 Configuration Frame DATA Structure

| Byte | Name | Meaning | Rule |
| --- | --- | --- | --- |
| DATA[0] | ctrl | Control byte: RW + Status + MainCommand. | Request: status bits must be 0. Response: status bits report result. |
| DATA[1] | subcmd | Subcommand / parameter index. | Selects the exact parameter under the MainCommand. |
| DATA[2..7] | payload | Parameter value or reserved bytes. | Little-endian. Read request uses zeros; write request carries the value. |

2.3 Broadcast Frame DATA Structure

| Byte | Meaning | Rule |
| --- | --- | --- |
| DATA[0..7] | Measurement fields defined by the PGN. | No ctrl byte and no subcmd byte. For example PGN 0xFF00 means DATA[0..5] are Roll/Pitch/Yaw raw values. |

2.4 Recommended Reading Order for Customers

Step 1: Check the CAN ID. If PGN is 0xEF00, parse it as a parameter configuration frame.

Step 2: If PGN is 0xFF00 to 0xFF06, parse it as a broadcast data frame according to the corresponding PGN table.

Step 3: For configuration frames, decode DATA[0] first, then DATA[1], then DATA[2..7].

Step 4: For broadcast frames, directly decode DATA[0..7] using the PGN field definition and scaling formula.

3. CAN Identifier Rules

3.1 Parameter Configuration Identifier, PGN 0xEF00

Parameter configuration uses J1939 Proprietary A style addressing. The destination address is placed in the PDU Specific field, and the source address is placed in the least significant byte.

| Direction | 29-bit ID Formula | Example when MCS10=0x7F, Master=0x00 |
| --- | --- | --- |
| Master -> MCS10 | ID = 0x18EF0000 ¦ (MCS10_Address << 8) ¦ Master_Address | 0x18EF7F00 |
| MCS10 -> Master | ID = 0x18EF0000 ¦ (Master_Address << 8) ¦ MCS10_Address | 0x18EF007F |

| ID Byte | Example Master -> MCS10 | Meaning |
| --- | --- | --- |
| 0x18 | Priority / data page field used by examples | Fixed in this document examples. |
| 0xEF | PDU Format | Indicates PGN 0xEF00, parameter configuration. |
| 0x7F | Destination Address | MCS10 ECU address. |
| 0x00 | Source Address | Master address. |

3.2 Broadcast Data Identifier

Broadcast data frames use the selected data PGN and the MCS10 ECU address as the source address. There is no destination address in the broadcast ID.

| Direction | 29-bit ID Formula | Example |
| --- | --- | --- |
| MCS10 Broadcast | ID = 0x18000000 ¦ (PGN << 8) ¦ MCS10_Address | Euler PGN 0xFF00 from MCS10 0x7F: 0x18FF007F |

4. Parameter Configuration Protocol (Privilege A)

4.1 Command Frame Format

| Byte | Field | Description | Example |
| --- | --- | --- | --- |
| 0 | ctrl | RW + Status + MainCommand. | 0x81 = Write + STATUS_OK + BasicParam. |
| 1 | subcmd | Parameter index under the selected MainCommand. | 0x05 = Output Enable. |
| 2-7 | payload | Parameter value, little-endian. Unused bytes shall be 0. | 0x07 0x00 = Euler + Accel + Gyro enabled. |

4.2 Ctrl Byte Definition

| Bit | Field | Mask | Meaning |
| --- | --- | --- | --- |
| 7 | RW | 0x80 | 0 = Read, 1 = Write. |
| 6-4 | Status | 0x70 | Valid mainly in response frames. Request frames shall set this field to 0. |
| 3-0 | MainCommand | 0x0F | 0 = Control, 1 = BasicParam, 2 = SystemParam. |

| Status | Decoded Meaning |
| --- | --- |
| 0x0 | STATUS_OK |
| 0x1 | STATUS_ILLEGAL_CMD response MainCommand has error. |
| 0x2 | STATUS_ILLEGAL_SUBCMD response SubCommand has error. |
| 0x3 | STATUS_ILLEGAL_PARAM response Param has error. |
| 0x4 | STATUS_REJECTED response lack of prerequisite configuration conditions. |

4.3 Request and Response Rules

| Operation | Request DATA | Successful Response DATA | Explanation |
| --- | --- | --- | --- |
| Read parameter | ctrl=Read, subcmd=parameter index, payload=0 | ctrl=Read + STATUS_OK, same subcmd, payload=value | Master reads one parameter value from MCS10. |
| Write parameter | ctrl=Write, subcmd=parameter index, payload=new value | ctrl=Write + STATUS_OK, same subcmd, payload=accepted value or echo | Master writes one parameter value to MCS10. |
| Control command | ctrl=Write + Control, subcmd=control index, payload=0 | ctrl=Write + STATUS_OK + Control, same subcmd | Control commands are write only. |
| Error response | N/A | ctrl contains non-zero status bits | Check status bits 6-4 to identify illegal command, illegal subcommand, illegal parameter, or rejected. |

Customer tip: for a configuration frame, always decode ctrl first. After ctrl is known, subcmd tells which parameter is being operated, and payload tells the value.

4.4 MainCommand List

| MainCommand | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | Control | Write only | Reset, save parameters, load defaults, apply address/baudrate change. |
| 1 | BasicParam | Read / Write | Common device parameters such as J1939 enable, SN, address, baudrate, work mode, output enable, output rate, LPF, termination resistor, install Euler angle. |
| 2 | SystemParam | Reserved / system use | System parameter group. |

4.5 Control Commands, MainCommand = 0

| subcmd | Command | Request DATA | Description |
| --- | --- | --- | --- |
| 0x00 | System Reset | 80 00 00 00 00 00 00 00 | Reset the device. |
| 0x01 | Instant Save Param | 80 01 00 00 00 00 00 00 | Save current parameters immediately. |
| 0x02 | Load Default Param | 80 02 00 00 00 00 00 00 | Restore default parameters. |
| 0x03 | Param Take Effect Instant | 80 03 00 00 00 00 00 00 | Apply ECU address, baudrate and termination resistor changes immediately. |
| 0x04 | Fusion Filter Reset | 80 04 00 00 00 00 00 00 | Reset Fusion Filter(DOZER) |

4.6 Basic Parameters, MainCommand = 1

| subcmd | Parameter | Data Length | Access | Value / Range | Read DATA Example | Write DATA Example |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00 | Enable J1939 | 1 byte | R/W | 0=Disable, 1=Enable | 01 00 00 00 00 00 00 00 | 81 00 01 00 00 00 00 00 |
| 0x01 | Device SN | 6 bytes | R/W | Device serial number, little-endian | 01 01 00 00 00 00 00 00 | 81 01 56 34 12 00 00 00 |
| 0x02 | ECU J1939 address | 1 byte | R/W | 0x01-0x7F | 01 02 00 00 00 00 00 00 | 81 02 80 00 00 00 00 00 |
| 0x03 | CAN baudrate | 4 bytes | R/W | 250000, 500000, 800000, 1000000 | 01 03 00 00 00 00 00 00 | 81 03 20 A1 07 00 00 00 |
| 0x04 | Work Mode | 1 byte | R/W | See 4.7.1 | 01 04 00 00 00 00 00 00 | 81 04 01 00 00 00 00 00 |
| 0x05 | Output Enable | 2 bytes | R/W | Bit mask, see 4.7.2 | 01 05 00 00 00 00 00 00 | 81 05 07 00 00 00 00 00 |
| 0x06 | Output Rate | 4 bytes | R/W | 1,5,10,20,50,100 Hz | 01 06 00 00 00 00 00 00 | 81 06 14 00 00 00 00 00 |
| 0x07 | LowPassFilter Config | 3 bytes | R/W | Order, cutoff, disable flag | 01 07 00 00 00 00 00 00 | 81 07 02 04 01 00 00 00 |
| 0x08 | Termination Resistor | 1 byte | R/W | 0=Disable, 1=Enable | 01 08 00 00 00 00 00 00 | 81 08 01 00 00 00 00 00 |
| 0x09 | Install EulerAngle | 6 bytes | R/W | Roll, Pitch, Yaw as int16 degrees | 01 09 00 00 00 00 00 00 | 81 09 5A 00 00 00 5A 00 |
| 0x0A | FusionSlaveID | 1 byte | R/W | 0x01-0x7F | 01 0A 00 00 00 00 00 00 | 81 0A 60 00 00 00 00 00 |
| 0x0B | HeartPeriod | 2 byte | R/W | Unit:ms | 01 0B 00 00 00 00 00 00 | 81 0B F4 01 00 00 00 00 |
| 0x0C | OperateStatus | 1 byte | R/W | 5: operate 127:pre-operate | 01 0C 00 00 00 00 00 00 | 81 0C 7F 00 00 00 00 00 |
| 0x0D | VendorID | 4 byte | R |  | 01 0D 00 00 00 00 00 00 |  |
| 0x0E | ProductCode | 4 byte | R |  | 01 0E 00 00 00 00 00 00 |  |
| 0x0F | InstallOrientation | 1 byte | R/W |  | 01 0F 00 00 00 00 00 00 | 81 0F 00 00 00 00 00 00 |

4.7 SystemParams, MainCommand = 2 (Multi-packet reading, TP transport protocol)

| subcmd | Command | Request DATA |
| --- | --- | --- |
| 0x00 | Firmware Version | 02 00 00 00 00 00 00 00 |
| 0x01 | System Config | 02 01 00 00 00 00 00 00 |

RTS/CTS peer-to-peer multi-packet transmission

4.7.1 Firmware Version

AHRS_H3_A3.0_f412b52ff68ec8253f0

Communication Process：

SendID J1939:18EF7F80 Data:02 00 00 00 00 00 00 00

RecvID J1939:1CEC807F Data:10 20 00 05 01 00 EF 00

SendID J1939:1CEC7F80 Data:11 01 01 FF FF 00 EF 00

RecvID J1939:1CEB807F Data:01 41 48 52 53 5F 48 33

SendID J1939:1CEC7F80 Data:11 01 02 FF FF 00 EF 00

RecvID J1939:1CEB807F Data:02 5F 41 33 2E 30 5F 66

SendID J1939:1CEC7F80 Data:11 01 03 FF FF 00 EF 00

RecvID J1939:1CEB807F Data:03 34 31 32 62 35 32 66

SendID J1939:1CEC7F80 Data:11 01 04 FF FF 00 EF 00

RecvID J1939:1CEB807F Data:04 66 36 38 65 63 38 32

SendID J1939:1CEC7F80 Data:11 01 05 FF FF 00 EF 00

RecvID J1939:1CEB807F Data:05 35 33 66 30 FF FF FF

SendID J1939:1CEC7F80 Data:13 20 00 05 FF 00 EF 00

4.7.2 System Config

Work Mode : 4

LowpassFilter: 0 3 0

Install Angle: 0 0 0

Device SN : 1234567

Sys Cfg : 6A 0 0

Data Rate : 100

Fusion Cfg : 7A

CAN Cfg : 500000 7F 0

CANopen TPDO1: 0 1FF 0 00000000 00000000 00000000 00000000

CANopen TPDO2: 0 2FF 0 00000000 00000000 00000000 00000000

CANopen TPDO3: 0 3FF 0 00000000 00000000 00000000 00000000

CANopen TPDO4: 0 4FF 0 00000000 00000000 00000000 00000000

J1939 Cfg : 1 00000017

ParaVersion : 1.0

4.8 Detailed Value Definitions

4.8.1 Work Mode Values

| Value | Mode | Description |
| --- | --- | --- |
| 0 | Basic Tilt | Basic tilt mode. |
| 1 | Dynamic Tilt | Recommended for construction machinery such as excavators. |
| 2 | Magnetic Heading | Magnetic heading mode. |
| 3 | Bulldozer | C-Frame / master mode. Output messages are configured automatically. |
| 4 | Bulldozer | Blade / slave mode. Output messages are configured automatically. |
| 5 | Dynamic Tilt, Smooth Convergence | Dynamic tilt mode with slow/smooth convergence. |

4.8.2 Output Enable Bit Mask

| Bit Mask | Message | PGN | Description |
| --- | --- | --- | --- |
| 0x0001 | Euler | 0xFF00 | Roll, Pitch, Yaw. |
| 0x0002 | Accel | 0xFF01 | Acceleration X/Y/Z. |
| 0x0004 | Gyro | 0xFF02 | Angular velocity X/Y/Z. |
| 0x0008 | Mag | 0xFF03 | Magnetic field X/Y/Z. |
| 0x0010 | Time | 0xFF04 | Time data. |
| 0x0020 | Quaternion | 0xFF05 | Attitude quaternion. |
| 0x0040 | Dozer | 0xFF06 | Swing angle data. |

Example: enable Euler + Accel + Gyro = 0x0001 + 0x0002 + 0x0004 = 0x0007. Because payload is little-endian, DATA[2]=0x07 and DATA[3]=0x00.

4.8.3 LowPassFilter Encoding

| Payload Byte | Name | Value | Meaning |
| --- | --- | --- | --- |
| DATA[2] | Order | 0x00 / 0x01 / 0x02 | Order 1 / 2 / 3. Default is 0x00. |
| DATA[3] | Cutoff | 0x00 | 0.08 Hz |
| DATA[3] | Cutoff | 0x01 | 0.16 Hz |
| DATA[3] | Cutoff | 0x02 | 0.32 Hz |
| DATA[3] | Cutoff | 0x03 | 0.64 Hz, default |
| DATA[3] | Cutoff | 0x04 | 1.28 Hz |
| DATA[3] | Cutoff | 0x05 | 2.56 Hz |
| DATA[3] | Cutoff | 0x06 | 5.12 Hz |
| DATA[3] | Cutoff | 0x07 | 10.24 Hz |
| DATA[4] | Disable LPF | 0x00 / 0x01 | 0 = enable LPF, 1 = disable LPF. |

4.8.4 InstallOrientation

| Value | Orientation |
| --- | --- |
| 0 | IO_TopFront |
| 1 | IO_TopRight |
| 2 | IO_TopBack |
| 3 | IO_TopLeft |
| 4 | IO_BottomFront |
| 5 | IO_BottomRight |
| 6 | IO_BottomBack |
| 7 | IO_BottomLeft |
| 8 | IO_RightFront |
| 9 | IO_RightTop |
| 10 | IO_RightBack |
| 11 | IO_RightBottom |
| 12 | IO_LeftFront |
| 13 | IO_LeftTop |
| 14 | IO_LeftBack |
| 15 | IO_LeftBottom |
| 16 | IO_FrontTop |
| 17 | IO_FrontRight |
| 18 | IO_FrontBottom |
| 19 | IO_FrontLeft |
| 20 | IO_BackTop |
| 21 | IO_BackRight |
| 22 | IO_BackBottom |
| 23 | IO_BackLeft |

IO_TopFront: Label facing up, X-axis pointing forward

IO_LeftTop: Label facing left, X-axis pointing up

4.9 Fully Annotated Configuration Examples

4.9.1 Write Output Enable = Euler + Accel + Gyro

| Item | Value | Explanation |
| --- | --- | --- |
| CAN ID | 0x18EF7F00 | Master 0x00 sends to MCS10 address 0x7F. |
| DATA | 81 05 07 00 00 00 00 00 | 8-byte command frame. |
| DATA[0] = 0x81 | Write + STATUS_OK + BasicParam | Bit7=1 means Write, bits6-4=0, bits3-0=1 BasicParam. |
| DATA[1] = 0x05 | Output Enable | Selects Output Enable parameter. |
| DATA[2..3] = 07 00 | 0x0007 | Enable Euler, Accel, and Gyro messages. |
| DATA[4..7] | 00 00 00 00 | Unused bytes set to 0. |

4.9.2 Write CAN Baudrate = 500000 bps

| Item | Value | Explanation |
| --- | --- | --- |
| CAN ID | 0x18EF7F00 | Master 0x00 sends to MCS10 address 0x7F. |
| DATA | 81 03 20 A1 07 00 00 00 | Write BasicParam / CAN baudrate. |
| Payload | 20 A1 07 00 | Little-endian 0x0007A120 = 500000 decimal. |
| After writing | 80 03 00 00 00 00 00 00 | Send Param Take Effect Instant when address or baudrate should take effect immediately. |

4.9.3 Read Install EulerAngle

| Frame | DATA | Explanation |
| --- | --- | --- |
| Request | 01 09 00 00 00 00 00 00 | Read BasicParam, subcmd 0x09. |
| Response example | 01 09 A6 FF 00 00 00 00 | Roll = int16 0xFFA6 = -90, Pitch = 0, Yaw = 0. |
| Response example | 01 09 5A 00 00 00 00 00 | Roll = int16 0x005A = 90, Pitch = 0, Yaw = 0. |

4.10 Parameter configuration notes

To use J1939, you must first enable J1939; otherwise, some parameters cannot be configured.

The following applies to MCS10 firmware version >= A2.4.

- Successfully setting up bulldozer mode will automatically configure the output parameters, eliminating the need for manual configuration changes, which could affect system operation.

- Modifying the address, baudrate or termination resistor will not take effect immediately; writing 0x03 in section 4.5 will make it take effect immediately.

5. Broadcast Data Protocol (Privilege B)

5.1 Broadcast Data Type and PGN

| Data Type | PGN | CAN ID Example, MCS10=0x7F | Description |
| --- | --- | --- | --- |
| Euler Angles | 0xFF00 | 0x18FF007F | Roll, Pitch, Yaw. |
| Acceleration | 0xFF01 | 0x18FF017F | X/Y/Z acceleration. |
| Gyroscope | 0xFF02 | 0x18FF027F | X/Y/Z angular velocity. |
| Magnetometer | 0xFF03 | 0x18FF037F | X/Y/Z magnetic field. |
| Time | 0xFF04 | 0x18FF047F | Time data. |
| Quaternion | 0xFF05 | 0x18FF057F | Attitude quaternion. |
| Dozer | 0xFF06 | 0x18FF067F | Swing angle data. |
| Sync | 0xFFFE | 0x18FFFE00 | Sync Time |
| HeartBeat | 0xFFFF | 0x18FFFF7F | HeartBeat msg |

5.2 Scaling and Physical Value Conversion

Unless otherwise specified, broadcast raw data is signed int16 in little-endian format. Convert raw values to physical values using the table below.

| Data Type | Scale | Physical Unit | Formula |
| --- | --- | --- | --- |
| Euler Angles | 180 / 32000.0 | degree | Euler_deg = int16_raw x (180 / 32000.0) |
| Quaternion | 1 / 32000.0 | None | Quaternion = int16_raw x (1 / 32000.0) |
| Acceleration | 8 / 32000.0 | g | Acc_g = int16_raw x (8 / 32000.0) |
| Gyroscope | 500 / 32000.0 | degree/sec | Gyro_dps = int16_raw x (500 / 32000.0) |
| Magnetometer | 500 / 32000.0 | uT | Mag_uT = int16_raw x (500 / 32000.0) |

Note: Install EulerAngle in BasicParam uses int16 degree values in the configuration payload. Broadcast Euler Angles use the scaling formula above.

5.3 Broadcast Data Field Definitions

5.3.1 Euler Angle, PGN 0xFF00

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | Roll | int16 | Roll angle raw value |
| 2-3 | Pitch | int16 | Pitch angle raw value |
| 4-5 | Yaw | int16 | Yaw angle raw value |
| 6-7 | Reserve | uint16 | Reserved |

5.3.2 Acceleration, PGN 0xFF01

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | AccX | int16 | X-axis acceleration raw value |
| 2-3 | AccY | int16 | Y-axis acceleration raw value |
| 4-5 | AccZ | int16 | Z-axis acceleration raw value |
| 6-7 | Reserve | uint16 | Reserved |

5.3.3 Gyroscope, PGN 0xFF02

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | GyroX | int16 | X-axis angular velocity raw value |
| 2-3 | GyroY | int16 | Y-axis angular velocity raw value |
| 4-5 | GyroZ | int16 | Z-axis angular velocity raw value |
| 6-7 | Reserve | uint16 | Reserved |

5.3.4 Magnetometer, PGN 0xFF03

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | MagX | int16 | X-axis magnetic field raw value |
| 2-3 | MagY | int16 | Y-axis magnetic field raw value |
| 4-5 | MagZ | int16 | Z-axis magnetic field raw value |
| 6-7 | Reserve | uint16 | Reserved |

5.3.5 Quaternion, PGN 0xFF05

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | Q0 | int16 | Scalar component raw value |
| 2-3 | Q1 | int16 | X component raw value |
| 4-5 | Q2 | int16 | Y component raw value |
| 6-7 | Q3 | int16 | Z component raw value |

5.3.6 Dozer Data, PGN 0xFF06

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-1 | Status | uint16 | Flag bits |
| 2-3 | BladeAngle | int16 | Blade angle raw value |
| 4-7 | Reserve | byte[4] | Reserved |

| Status |  |  |  |
| --- | --- | --- | --- |
| No. | Name | Bit Mask | Meaning |
| 1 | Init | 0X0001 | Heading has been established before |
| 2 | Valid | 0X0002 | Current heading is valid |

5.3.7 HeartBeat, PGN 0xFFFF

| Byte Offset | Parameter | Type | Description |
| --- | --- | --- | --- |
| 0-0 | Status | Uint8 | Operate status |
| 1-7 | Reserve | byte[7] | Reserved |

5.4 Fully Annotated Broadcast Example

| Item | Value | Explanation |
| --- | --- | --- |
| CAN ID | 0x18FF007F | PGN 0xFF00 = Euler Angle, source address = 0x7F. |
| DATA | 00 40 00 00 00 00 00 00 | Euler broadcast data, not a configuration command. |
| Roll raw | 0x4000 = 16384 | Physical roll = 16384 x 180 / 32000 = 92.16 degree. |
| Pitch raw | 0x0000 | Physical pitch = 0 degree. |
| Yaw raw | 0x0000 | Physical yaw = 0 degree. |

6. Quick Configuration Procedures

6.1 Enable J1939 Output

Send BasicParam / Enable J1939 write command: CAN ID = 0x18EF7F00, DATA = 81 00 01 00 00 00 00 00.

Configure Output Enable according to required broadcast types. Example Euler + Accel + Gyro: DATA = 81 05 07 00 00 00 00 00.

Configure Output Rate. Example 20 Hz: DATA = 81 06 14 00 00 00 00 00.

Save parameters if required: DATA = 80 01 00 00 00 00 00 00.( Optional, as the system will automatically save the parameters after 3 seconds. )

6.2 Change ECU Address or Baud Rate

Write the new ECU address or CAN baudrate using BasicParam.

Send Param Take Effect Instant: DATA = 80 03 00 00 00 00 00 00.

After the setting takes effect, use the new address or baud rate for subsequent communication.

6.3 Recommended Customer Debug Checklist

| Check Item | Expected Result |
| --- | --- |
| CAN frame type | Extended 29-bit frame, DLC = 8. |
| Configuration request ID | For default address, master to MCS10 should use 0x18EF7F00. |
| Configuration response ID | For default address, MCS10 to master should use 0x18EF007F. |
| Read request payload | DATA[2..7] should be zero. |
| Write request payload | DATA[2..7] should contain the little-endian value. |
| Response status | Status bits in DATA[0] should be 0 for success. |
| Broadcast parsing | Use PGN field table. Do not parse broadcast DATA[0] as ctrl. |

7. Bulldozer Mode Application

The following applies to MCS10 firmware version >= A2.6.

Prerequisites: baud rate 500 kbps, J1939 enabled.

- Configure the blade sensor: work mode to 0x4, configure the FusionID as the C-Frame sensor ID.

- Configure the C-Frame sensor: work mode to 0x3.

- Configure the body sensor: work mode to 0x1, and configure the output data and output rate.